# RISC Architecture

## History and Development

# Outline

## what is RISC?

## General features of RISC microprocessors

## RISC vs. CISC – the history of the debate

## Performance Equation

## RISC vs. CISC – the differences

## RISC–CISC convergence?

## RISC/Post-RISC  µPs today

# what is risc?

## RISC: Reduced Instruction Set Computer

### examples: Sparc, MIPS, PowerPC

## CISC: Complex Instruction Set Computer

### examples: Intel architectures and most earlier µPs, microcontrollers, mainframes

# general features

## one cycle execution time

### CPI (clock per instruction) of 1 cycle

## simple instruction decode logic

### LOAD/STORE

## pipelining

### simultaneous execution of parts/stages

## large number of registers

# RISC vs. CISC

## RISC vs. CISC debate

### not so much technologies as design strategies

### each an approach to different problems encountered historically

### different problems  different solutions

# the case for cisc

## Problems faced:

### Storage and Memory

#### expensive and slow

#### “Good Code = COMPACT Code”

### Compilers

#### HLL  assembly language

#### Assembly language  (assembler) machine code

# the cisc solution

## Close the “Semantic Gap”

### Increase the complexity of instructions so they more closely match the HLL constructs

### Complex instructions = complex sequence of operations over many clock cycles

## Microcoding

## (complexity: software  hardware)

# the performance equation: cisc

## Time/program = (instructions/program) * (cycles/instruction) * (time/cycle)

## CISC:

### reduce instructions/program

### cost: increased cycles/instruction

# risc design philosophy

## No instruction or addressing mode whose function can be implemented by a sequence of other instructions should be included in the ISA unless its inclusion can be qualitatively shown to improve performance by a non-trivial amount…

# risc design philosophy

## Secondary axiom:

## a RISC processor should not have to do anything at run time in hardware that could instead be done at compile time in software.

# the performance equation: risc

## Time/program = (instructions/program) * (cycles/instruction) * (time/cycle)

## RISC:

### reduce cycles/instruction

### cost: increased instructions/program

# the case for risc

## Patterson: “make the common case fast”

## Many of the problems which led to CISC implementations no longer exist

### Improved compiler technology

### Memory getting cheaper

## HLL support could be better done in software

# risc vs. cisc – the differences

## CISC

## Emphasis on hardware

## Multi-clock complex instructions

## Memory-to-memory: “LOAD”and “STORE” incorporated in instructions

## Small code sizes, high cycles per second

## Transistors used for storing complex instructions

## RISC

## Emphasis on software

## Single-clock, reduced instruction only

## Register-to-register: “LOAD”and”STORE” are independent instructions

## Low cycles per second, large code sizes

## Spends more transistors on memory registers

# risc vs cisc - example

## Multiply 2 Numbers in Memory

### Multiply 2:3, 5:2

### Execution unit carries out all computations

### Execution unit can only operate on data in registers

## CISC: complex instruction

### “MULT 2:3, 5:2”

## RISC: simple instructions

### LOAD A, 2:3

### LOAD B, 5:2

### PROD A, B

### STORE 2:3, A

# general features (risc)

## one cycle execution time

### CPI (clock per instruction) of 1 cycle

## simple instruction decode logic

### LOAD/STORE

## pipelining

### simultaneous execution of parts/stages

## large number of registers

# advantages of risc

## one-cycle execution time

### longer code approximates same amount of time as multi-cycle “MULT”

### reduced instructions require less transistors of hardware space = more room for general purpose registers

## pipelining possible

### more instructions can be executed in a shorter period of time

# advantages of risc

## Higher performance

### Simple instruction  Faster circuits

#### A complex instruction set makes all instructions slower

#### Therefore: make basic instructions simple

#### If required, synthesize more complex instructions from the basic set

##### Instruction sequences are slower, but on average, the program will run faster

# drawbacks of risc

## RISC machines have poorer code density

### Fixed length inefficient

#### Larger memory size (greater cost)

#### More memory accesses (greater power consumption)

## Lack of software support

### Intel

### Backwards compatibility

# risc-cisc convergence?

## Increased processor speeds

### CISC chips able to execute more than 1 instruction within a single clock

### CISC chips can do pipelining

## More transistors on a chip

### RISC processors have space to implement more complex commands

## Superscalar architecture

### Out-of-order execution (on RISC hardware)

### Branch prediction

# superscalar architecture

## Superscalar processing complicates CPU design due to need to manage out-of-order and branch instructions

## Branch instructions may be executed speculatively or discarded if incorrect

## Average instruction time is reduced by pipelining and superscalar processing, not the execution time of individual instructions

## Used in both RISC and CISC